### Automation

Automation is the use of m/c control systems and information technologies to increase/ optimise productivity for the production of the goods and delivery of services.

Automation:

- Fixed or Hard automation(Ex: sewing m/c, electric iron, fan)

  - Adjustable automation (Ex: adjust stitch length, pitch of screw of a lathe m/c etc.)

- Programmable automation after arrival of computers, NC & CNC m/c, CAM & CIM etc.

- Flexible automation

Robot is a computer control m/c which is coming under the category of Highest level of autonomy/automations.

### Fixed Automation

A manufacturing system in which the sequence of processing (or assembly) operations is fixed by the equipment configuration

Typical features:

- Suited to high production quantities

- High initial investment for custom-engineered equipment

- High production rates

- Relatively inflexible in accommodating product variety

Programmable Automation

A manufacturing system designed with the capability to change the sequence of operations to accommodate different product configurations

Typical features:

  - High investment in general purpose equipment

  - Lower production rates than fixed automation

  - Flexibility to deal with variations and changes in product configuration

  - Most suitable for batch production

  - Physical setup and part program must be changed between jobs (batches)

## Flexible Automation

An extension of programmable automation in which the system is capable of changing over from one job to the next with no lost time between jobs

Typical features:

- High investment for custom-engineered system

- Continuous production of variable mixes of products

- Medium production rates

- Flexibility to deal with soft product variety